# Smart IoT Pet Feeder – Final Report

# 1. Introduction

This project presents a Smart IoT Pet Feeder system designed to detect pet presence, automatically dispense food, and report telemetry using MQTT and Blynk. It supports remote interaction and real-time monitoring through mobile and cloud-based systems(mqtt telemetry to mqtt explorer).

# 2. Hardware and Electronics

The system uses the following Grove components:
- PIR Motion Sensor
- Ultrasonic Ranger
- Servo Motor
- LED
- Buzzer
- LCD Display (optional)
- Fan (powered directly by 5V/GND)

GPIO Mapping:
- PIR Sensor: GPIO 16 (D16) – input
- Ultrasonic Sensor: GPIO 18 – distance measurement
- Servo Motor: GPIO 24 – PWM for flap control
- LED: GPIO 5 – output indicator
- Buzzer: GPIO 22 – feedback during feeding
- LCD: I2C (SDA/SCL)
- Fan: 5V (Pin 2) and GND (Pin 6)

# 3. Software and Code

The pet feeder system relies on two key Python scripts: one for the IoT device (Raspberry Pi), and one for the remote server (PC/laptop) that simulates cloud-side logic. These scripts interact through MQTT.

pet_feeder.py – Main IoT Script (Raspberry Pi)

This script runs directly on the Raspberry Pi and performs all real-world interactions. Its responsibilities include:

- Initializing GPIO pins for all connected components:

  - PIR motion sensor to detect presence

  - Ultrasonic ranger to measure distance to the pet

  - LED and buzzer for feedback

  - Servo motor for dispensing food

  - LCD display for system messages

- Setting up a PWM signal to control the servo motor’s angle (open/close flap).

- Interfacing with Blynk, a mobile IoT dashboard, using virtual pins:

  - V0 = Enable/Disable feeder system

  - V1 = Motion status

  - V2 = Distance display

- Reading sensor values and publishing JSON telemetry to test.mosquitto.org on topic oisin123/telemetry.  Each message contains:

  - "motion" (boolean)

  - "distance_cm" (float)

  - "timestamp" (string)

- Displaying live values on the LCD (distance, system status).

- Running logic to prevent false triggers:

  - Only triggers feeding when motion is detected and the distance is below 40 cm twice in a row.

- Choosing between:

  - big_feed() (initial food drop) and

  - small_feed() (shorter repeat feed after cooldown).

pet_feeder_server.py – Remote Listener Script (PC/Simulated Cloud)

This script runs on a laptop or PC and represents the cloud or backend logic. It performs the following:

- Subscribes to oisin123/telemetry and listens for incoming sensor data from the Pi.

- Parses incoming JSON messages and checks:

  - Is "motion" set to true?

  - Is "distance_cm" less than 40?

- If both conditions are met, it sends a feed command:

  - Publishes {"feed_now": true} to the oisin123/commands topic.

- The Raspberry Pi receives this message and triggers either a big or small feed depending on cooldown state.

Technologies Used

- RPi.GPIO – for direct hardware control

- grove.gpio – for Grove-compatible sensors (e.g., ultrasonic ranger)

- paho-mqtt – for MQTT publish/subscribe

- BlynkLib – for mobile app control and feedback

- json, time – for data formatting and delays

- JHD1802 – for controlling the I2C 16x2 LCD screen

📸 Screenshot 1: MQTT & Blynk Setup
Caption:
Initialises MQTT connection to test.mosquitto.org and links Blynk virtual pins for remote control (V0 = enable, V1 = motion, V2 = distance).

# 📸 Screenshot 2: GPIO & Hardware Setup
Caption:
Maps each hardware component (PIR, LED, buzzer, servo, ultrasonic) to a specific GPIO pin. Sets up servo PWM and sensor initialization.

# Screenshot 3: Feeding Functions
Caption:
Defines big_feed() and small_feed() functions for different food portions. Activates servo, LED, buzzer, and updates LCD screen.

# 📸 Screenshot 4: Main Loop & Feeding Trigger 
Caption:
Main decision loop: checks motion + distance, publishes telemetry to MQTT, updates Blynk and LCD, and triggers feeding after 2 detections.

# Screenshot 5: Server-Side MQTT Trigger
Caption:
Listens to telemetry from the Pi. If motion is true and distance is below 40 cm, sends a command (feed_now: true) back to the Pi via MQTT.

# 4. Challenges and Solutions

- Servo jitter resolved by resetting PWM with ChangeDutyCycle(0)
- Blynk unresponsiveness reduced by limiting time.sleep() usage and using updated import 
- Telemetry delay fixed by improving message structure and MQTT timing

# 5. Demo Video

A short demo video has been recorded and uploaded to YouTube. []